Containment Berm

Type – Continuous Air Wall Construction    (or air sides / foam ends drive thru models)

Your air wall containment berm comes equipped with two types of fill valves, a tire valve and a boat valve. It also has a 1.0 lb. pressure relief valve installed. Your air wall berm should NEVER be filled to over 1.0 lb. psi.

Instructions –

- Remove the berm from its storage bag and place in your desired location. Unfold and spread the berm out removing all folds or wrinkles in the floor area being careful not to move the berm in a dragging motion. Never drag the berm to relocate.

- Once the berm is spread out you may fill the air wall with any available air source, compressed air, leaf blower, etc. The air wall has a 1.0 lb. psi maximum fill pressure, NEVER over inflate your berm. The pressure relief valve is designed for expansion of air molecules due to heat and very minor over inflation. You should never connect an air supply to the berm and walk away this could damage the air wall and void any warranty.

- If you have an air wall sides, foam end wall model the foam end walls may be driven over. In case of a spill or containment procedure the foam walls will rise with the level of the liquid being contained.

- When finished with your berm you can deflate the air walls using the boat valve. Depress the center plunger and turn, it should catch and remain open. To remove all the air for safe storage you can vacuum the air out from the boat valve. If a vacuum is not available make sure you fold the berm towards the outlet boat valve removing as much air as possible. Also always fold the air walls to the inside of the berm so no areas of the air wall are exposed after folding. This will decrease the chance of  any scraping or damage to the air wall.

NOTE: If the berm is left up and inflated overnight or for long periods and there has been a decrease in temperature it may look as though the air wall has lost air. This is due the contraction of air molecules. You may have to partially inflate the berm. When the temperature rises the air molecules will expand at that time is when the pressure relief will activate and protect you berm from damage.

CAUTION! Your berm will become slick when wet. Always take care when walking on a wet berm!

Husky Portable Containment 1-800-260-9950/918-333-2000

“Leader of the Pack”

www.huskyportable.com